# Hardware Integrity & Actuation Protocol: The "Physical Hand"

## 1. The Actuation Loop: Beyond the "Worksheet"

The DEM (Worksheet) provides the instruction, but the Hub must provide the Verification. For the June 29th Trial, "Intent" isn't enough; we need "Execution."

### 1.1 Closed-Loop Feedback (The Pulse)

When the Hub executes a command from the DEM (e.g., OPEN_VALVE_A), it does not assume success. It initiates a three-stage hardware check:

- Electronic Confirmation: Verify the solenoid/actuator drew the expected current (detects wire cuts or blown fuses).

- Hydraulic Confirmation: Verify a change in the Telemetric Endpoint (Flow Meter) data within 30 seconds of actuation.

- State-Locking: Only after hydraulic confirmation is the event written to the IRP Ledger.

## 2. The "Handshake" of the Three Nodes

To guarantee the hardware works, we rely on a specialized communication hierarchy:

| Device | Responsibility | Hard-Link Guarantee |
| --- | --- | --- |
| Sensor (TE) | Raw Acquisition | Uses Sub-GHz Long-Range to punch through dense crop canopies where Wi-Fi/Bluetooth fail. |
| Relay (DAN) | Concentration | Maintains a Local Volatile Buffer. If the Hub is busy, the Relay holds the data so no "moisture spikes" are missed. |
| Hub (ECG) | The Executioner | Equipped with a Supercapacitor/Battery Backup. If the farm loses power, the Hub has enough juice to "Safe State" the valves and write the final log to flash. |

## 3. Hardware "Sovereignty" (The Anti-Tamper)

If a regulator claims the farmer manually bypassed the system to steal water, the hardware must prove otherwise.

- Physical Seal: The Hub enclosure uses an internal light sensor or magnetic reed switch. If the box is opened, a "Tamper Event" is immediately hashed into the IRP Ledger.

- Manual Override Detection: The Hub monitors the valve state. If the valve opens without a DEM command, the Hub logs an "Unauthorized Actuation" event. This protects the QCV Report's integrity.

## 4. The "Worksheet" Execution Environment

The Hub doesn't run a full OS like Windows; it runs a Minimalist Deterministic Runtime.

- Logic Isolation: The actuation code is separated from the communication code. Even if the LTE modem crashes, the valve-control logic continues to pulse.

- Watchdog Timers: A hardware "Heartbeat" monitor. If the software freezes for more than 500ms, the hardware physically resets the processor to a known good state.

## 5. Deployment Reality Check (The "Mud" Factor)

To ensure this works in Monte Vista, CO:

- Thermal Management: Components rated for -40°C for winter storage and +85°C for summer heat inside a poly-box.

- Surge Protection: Every sensor line is optically isolated or TVS-diode protected against the San Luis Valley's lightning/static.