# Functional Requirements Document: Edge Computing Gateway (ECG)

## 1. Hardware Architecture & Performance

The Edge Computing Gateway (ECG) must serve as the localized execution node for the Deterministic Execution Manifest (DEM). It is designed for high-reliability operations in environments where network backhaul is intermittent.

### 1.1 Compute & Storage

- Local Runtime Environment: Must support a lightweight execution engine (e.g., Python-based or C++ micro-runtime) capable of parsing and executing JSON-serialized DEM decision trees.

- Persistent Cache: Minimum 4GB of industrial-grade flash storage to maintain a local buffer of the Spatial Query Engine (SQE) data and a 30-day rolling log of Immutable Resource Provenance (IRP) data.

- Cryptographic Module: Dedicated hardware-based Root of Trust (RoT) or Secure Element for manifest decryption and state-synchronization hashing.

### 1.2 Connectivity & Networking

- Primary Backhaul: Integrated 4G/5G LTE with an external high-gain antenna port.

- Local Mesh/LPWAN: Integrated LoRaWAN or proprietary Sub-GHz radio for communication with Telemetric Endpoints (sensors) and Data Aggregation Nodes (relays).

- Failover Logic: Automatic detection of backhaul loss, triggering immediate transition to "Autonomous Manifest Mode."

## 2. Software & Protocol Requirements

### 2.1 Manifest Ingestion & Validation

- Handshake Protocol: The ECG must perform a mutual TLS handshake with the Inference Engine before accepting a new DEM.

- Integrity Check: Upon receipt of a DEM, the ECG must verify the cryptographic signature against the stored public key.

- Temporal Sync: Must maintain an internal Real-Time Clock (RTC) synced via NTP (when connected) or GPS pulse-per-second to ensure Temporal Bound Parameters of the DEM are strictly enforced.

### 2.2 Local Inference Execution

- Threshold Monitoring: The ECG shall sample data from Telemetric Endpoints at a frequency defined within the DEM (e.g., every 15 minutes).

- Actuation Logic: Must provide direct GPIO or industrial protocol (Modbus/CAN bus) outputs to trigger physical hardware based on the DEM decision tree.

- Delta Calculation: Must calculate the variance between "Predicted Environmental State" (from the DEM) and "Actual Measured State" for later reporting.

## 3. Data Integrity & Reporting

### 3.1 IRP Ledger Generation

- Local Logging: Every actuation event must be logged with a timestamp, the triggering sensor value, and the unique ID of the governing DEM.

- Hashing: Each log entry must be chained to the previous entry using a SHA-256 hash to ensure the immutability of the field record.

### 3.2 Asynchronous State Synchronization

- Packetization: Upon restoration of network backhaul, the ECG must package all execution logs into a State Verification Packet.

- Priority Queue: The ECG must prioritize the transmission of Quantifiable Conservation Validation (QCV) data over non-critical system health telemetry.

## 4. Power & Environmental Specs

- Operating Temperature: -40°C to +85°C (Industrial Grade).

- Power Management: Must support solar-plus-battery input with ultra-low-power "Sleep" states between telemetric sampling windows.

- Enclosure: IP67-rated weatherproofing for deployment in exposed agricultural or industrial environments.